| Ain Shams University ASU Racing Team Faculty of Engineering |  |
| --- | --- |

# Path Planning Task

Prepared By: Seif Ahmed Mahrous Abdelnafea

Submission Date: 2/10/2026

# What is the Task About

This task is based on the Formula Student AI competition, but a simplified version of it. We get the car's position and heading (x, y, yaw) and a few cones around it. Blue cones are on the left side of the track, yellow ones are on the right. What we need to do is return a list of (x, y) points that make up a path the car can drive while staying between the cones.

Part 1 is about handling up to 2 cones on each side. Part 2 asks what we'd do if we had 3 cones on one side of the track, and we also had to add new test scenarios for that case.

# Overall Idea

The idea behind the algorithm is pretty simple. If there's a blue cone on the left and a yellow one on the right, the middle of the track is just the midpoint between them. So we find those midpoints, connect them into a smooth line, and then we simulate the car driving along that line so the output path is something the car can actually follow.

Here are the steps:

1. Move everything into the car's local frame so the car is at the origin facing forward.

2. Find the centre of the track based on what cones are available.

3. Order the centre points and smooth the line with Chaikin's corner cutting.

4. Simulate the car following that line using a pure pursuit controller.

5. Move the path back to world coordinates.

# Working in the Car's Frame

The first thing we do is transform all the cone positions into the car's own coordinate system. This makes the math easier because now the car sits at (0,0) and faces along the positive x-axis. So if we want to check whether a cone is in front of the car, we just check if x > 0. If it's to the left, y > 0.

We do this by subtracting the car's position from each cone and then rotating by the negative of the yaw angle with a 2D rotation matrix. When we're done building the path, we reverse the transformation to get back to world coordinates.

The methods for this are _to_local() and _to_world().

# Finding the Centre of the Track

This is the main part. How we figure out the centreline depends on what cones we can see:

## Both blue and yellow cones visible

This is the best situation. We go through the side that has more cones and for each one we find the closest cone of the other color. The midpoint between each pair is a centre point. We also get the track direction at each point, which is perpendicular to the line between the two cones in the pair.

## Only one color, 2 cones

When we only see cones on one side, we can still get the track direction from the line between those two cones. Then we push each cone inward by 1 meter (half the assumed track width) to estimate where the centre is. For blue cones that means pushing to the right, for yellow it's to the left.

## Only one cone

With just one cone there's no direction info, so we assume the track goes wherever the car is already pointing. We offset the cone 1 meter inward and use that as our centre point.

## No cones

No cones means we just go straight. There's nothing else to go on.

# Ordering and Smoothing

After we have the centre points, they need to be put in order so the path makes sense. The _chain() function handles this. It starts from the car, picks the nearest centre point that's in front of the current direction, moves there, and repeats. If a point would make the path go backwards it gets skipped.

We also add an "entry point" a bit before the first centre point. This way the car can line up with the track before it gets to the first gate, instead of cutting across diagonally.

Then we smooth the path using Chaikin's corner cutting. It's a pretty standard technique: you split each segment into smaller pieces, then replace each pair of points with two new points at 25% and 75% along the segment. Do this 3 times and the sharp corners turn into nice curves without changing the overall shape too much.

# Pure Pursuit - Making the Path Drivable

This is probably the most important step. The centreline we got from the cones doesn't know anything about where the car is actually pointing. Like in scenario 14, the car faces almost completely away from the track, so a path going straight to the midpoints would need an instant 120 degree turn, which a car can't do.

So we don't just return the centreline. Instead we simulate a car model driving along it using pure pursuit. Here's how it works:

• The car starts at its real position and heading.

• It picks a target point about 1 meter ahead on the centreline.

• It calculates steering with the pure pursuit formula: curvature = 2*sin(a) / distance, where a is the angle between the car's heading and the target.

• Steering is capped so the car can't turn tighter than a 1 meter radius.

• The car moves forward 5 cm, saves its new position, and repeats.

• We save a path point every 0.25 meters.

What this gives us is a path that starts in whatever direction the car is facing and curves toward the track gradually. If the car points the wrong way, it does a U-turn. The path always ends up between 7 and 10 meters long.

# Three Cones on One Side

## The problem

Two cones on one side only let you draw a straight line through them, so you'd think the track is straight. But three cones that aren't in a line tell you the track is curving. The question is how to use that info.

## What we did: circle fitting

Three points that aren't on a line define exactly one circle. So we compute the circle that goes through all three cones. That gives us how much the track is curving and in which direction.

To figure out whether the track bends left or right, we take the cross product of the vectors between consecutive cones. Positive means left, negative means right.

Then we offset that circle inward by half the track width to get the centreline. If we can see cones of the other color too, we measure the actual track width from them instead of just assuming 2 meters.

We follow the offset circle from the first cone past the last one, and keep going for another 90 degrees to predict where the track continues. This arc becomes our centreline, and the pure pursuit controller drives along it same as before.

If the three cones are almost in a straight line (radius bigger than 50m), we just fall back to the normal straight-line approach since there's basically no real curve to model.

## Why a circle

A circle is the simplest curve that can bend. Three points give exactly one circle, no fitting or tuning needed, just a direct formula. And corners on cone tracks are usually built as constant-radius arcs, so a circle fits the situation well.

We thought about other options too. A parabola depends on how you orient the axes and doesn't work well for tight or U-shaped turns. A spline passes through the points but gives bad results when you try to extend it past the last cone. A straight line just throws away the curvature information which is the whole reason we have a third cone.

## Limitations of this approach

The circle can only model constant curvature. It doesn't handle S-bends or corners that get tighter or wider as you go. It's also sensitive to noise in the cone positions when the cones are close together - a few cm of error can change the radius a lot. And the 90 degree extension past the last cone is just a guess. If the track straightens out after that we wouldn't know until we see new cones.

# New Test Scenarios

We added 4 new scenarios (21 to 24) for testing Part 2:

| Scenario | Cones | What It Tests |
| --- | --- | --- |
| 21 | 3 blue, curving left | Left turn with only the inner edge visible |
| 22 | 3 yellow, curving right | Right turn, inner edge only, plus the extrapolation |
| 23 | 3 yellow in a straight line | Collinear cones - should fall back to straight line method |
| 24 | 3 blue + 2 yellow | Track width measured from the other side instead of assumed |

# Code Structure

All the code is in the src/ folder:

| File | What It Does |
| --- | --- |
| models.py | Data types: Cone, CarPose, Path2D |
| path_planning.py | The main algorithm, everything described above |
| scenarios.py | All 24 test scenarios with cone positions and car poses |
| tester.py | Matplotlib visualizer for plotting cones, car, and path |
| run.py | Command line script to run a single scenario |

# Main Functions in path_planning.py

generatePath() - Entry point. Transforms cones to local frame, picks the circle or midpoint method, builds centreline, runs pure pursuit, converts back to world coords.

_cones_local() - Splits cones into blue and yellow lists and transforms them to the car's local frame. Sorts by distance from car.

_centre_waypoints() - Part 1 logic. Pairs cones, finds midpoints, offsets single-side cones. Returns ordered centre points and direction.

_curve_from_three_cones() - Part 2 logic. Fits a circle through 3 cones, offsets it, generates the arc.

_chain() - Orders centre points by going to the nearest one ahead each time. Stops the path from doubling back.

_smooth() - Chaikin corner cutting. Subdivides and rounds corners.

_pure_pursuit() - Simulates the car driving along the smoothed centreline with a limited turn radius. Gives us the final path.

_circle_through() - Calculates the circle through 3 points. Returns centre and radius, or None if they're collinear.

# Parameters

The tunable constants are at the top of path_planning.py so they're easy to change:

HALF_TRACK_WIDTH = 1.0 m: Distance from cone to centreline when we only see one side

STEP = 0.25 m: Gap between output path points

MIN/MAX_PATH_LENGTH = 7/10 m: How long the path can be

MIN_TURN_RADIUS = 1.0 m: Tightest turn the car can make

LOOKAHEAD = 1.0 m: How far ahead pure pursuit looks

SMOOTH_ITERATIONS = 3: Number of Chaikin smoothing passes

MAX_ARC_ANGLE = 90 deg: How far the Part 2 circle extends past the last cone

# Assumptions

• Track width is 2 meters (1 meter from cones to centreline). When we see both colors we measure it, otherwise we use this default.

• Cone colors are correct. A blue cone means left side of the track even if it shows up to the right of the car. That just means the car has to turn.

• We use all cones including ones behind the car. We tried ignoring them but that broke scenario 14 where the cones are behind the car's starting direction.

• If there's only one cone and no direction info, we assume the track goes along the car's current heading.

• Minimum turn radius is 1 meter. Real FS cars turn at about 4-5 meters but the scenarios here use a smaller scale so we scaled it down to match.

# Results

We tested all 24 scenarios and they all pass. Path lengths are between about 6.75 m and 9.75 m, step sizes are 0.25 m (well under the 0.5 m limit). The car never turns tighter than the 1 m minimum radius, and the path stays between the blue and yellow boundaries in every case.

The image in docs/all_scenarios.png shows results for all 24 scenarios. Green is the path, the red dot and arrow are the car, blue and yellow dots are cones.

# Limitations

• In most scenarios the car starts outside the track so the path has to enter it first. Sometimes the car passes close to a cone because of this.

• Pure pursuit cuts corners a bit on tight turns. A real car would do the same.

• The nearest-cone pairing can produce weird matches if cones are in unusual positions. Something like Delaunay triangulation would handle that better.

• No noise handling. We trust cone positions exactly as they are.

• Circle fit only works for constant curvature. Can't model S-bends or corners that change radius.

• We only plan a short path (about 7-10 m) from whatever cones are currently visible.